BIONIC LIMB SIMULATOR

Unity EMG-Driven Hand Demo

AI Agent Project Specification

Version 1.0

Target Engine: Unity 6000.3.10f1

March 2026

# Table of Contents

# 1. Project Overview

This document is a complete, self-contained specification for an AI coding agent to implement a bionic hand simulator in Unity 6000.3.10f1. The simulation drives a rigged 3D hand model using electromyography (EMG) signal data captured from a forearm myoband, decoded through a gesture classification pipeline.

The agent should treat this document as the single source of truth. Every section includes relevant documentation references. The agent must not hallucinate API calls; it must consult the linked documentation for any Unity, ML, or signal-processing API it is uncertain about.

## 1.1 Goals

- Simulate anatomically accurate hand movement driven by 8-channel surface EMG data.

- Classify EMG gesture patterns using an ML model (Unity Sentis + ONNX), with a rule-based fallback.

- Provide a minimal, functional Unity UI for file import, playback control, and step navigation.

- Define a synthetic myosignal file format (.myo.csv) that allows test data generation.

## 1.2 Out of Scope

- Real-time hardware streaming (live myoband connection).

- Haptic feedback systems.

- Full finger-tip tactile simulation.

- Online/cloud ML inference.

## 1.3 Key Documentation References

The agent must refer to the following primary sources throughout implementation:

| Domain | Resource | URL |
| --- | --- | --- |
| Unity 6 Scripting API | Unity Manual | https://docs.unity3d.com/6000.0/Documentation/ScriptReference/ |
| Unity Sentis (ML inference) | Sentis Documentation | https://docs.unity3d.com/Packages/com.unity.sentis@2.1/manual/ |
| ONNX Model Format | ONNX Specification | https://onnx.ai/onnx/intro/ |
| Hand Anatomy | Gray's Anatomy (hand) | https://www.bartleby.com/107/ |
| Forearm Muscle Anatomy | NCBI — Forearm EMG | https://www.ncbi.nlm.nih.gov/books/NBK526sprott/ |
| EMG Signal Processing | BioSig Processing Library | https://biosig.sourceforge.net/ |
| NinaPro EMG Dataset | NinaPro Database | https://ninapro.hevs.ch/ |
| Unity Animator / Rigging | Animation Rigging pkg | https://docs.unity3d.com/Packages/com.unity.animation.rigging@1.3/manual/ |
| Unity UI Toolkit | UI Toolkit Manual | https://docs.unity3d.com/6000.0/Documentation/Manual/UIElements.html |

# 2. Hand Anatomy & Biomechanics Model

The simulated hand must reflect the anatomical structure of the human hand as defined in Gray's Anatomy (https://www.bartleby.com/107/) and the Terminologia Anatomica standard. All bone and joint names used in code must match these anatomical conventions.

## 2.1 Skeletal Structure

The hand skeleton is composed of 27 bones divided into three groups:

- Carpals (8): Scaphoid, Lunate, Triquetrum, Pisiform, Trapezium, Trapezoid, Capitate, Hamate — forming the wrist. These are not individually animated in this demo but define the base transform anchor.

- Metacarpals (5): MC1–MC5, one per digit. MC1 (thumb) has a saddle joint enabling opposition.

- Phalanges (14): Each finger has Proximal (PP), Middle (MP), Distal (DP) phalanges. The thumb has only PP and DP.

## 2.2 Joints and Degrees of Freedom

Reference: Levangie & Norkin, Joint Structure and Function, Chapter 11 (hand). Each joint must be modelled with its anatomical range of motion (ROM):

| Joint | Bones | Axes | Flexion ROM | Abduction ROM |
| --- | --- | --- | --- | --- |
| CMC1 (thumb) | Trapezium–MC1 | 2 (saddle) | 0°–50° | 0°–40° |
| MCP1 (thumb) | MC1–PP1 | 1 (hinge) | 0°–60° | N/A |
| IP1 (thumb) | PP1–DP1 | 1 (hinge) | 0°–80° | N/A |
| MCP2–5 | MC–PP | 2 (condyloid) | 0°–90° | ±20° |
| PIP2–5 | PP–MP | 1 (hinge) | 0°–100° | N/A |
| DIP2–5 | MP–DP | 1 (hinge) | 0°–90° | N/A |

NOTE: The agent must clamp all joint rotations to the anatomical ROM values in the table above. Exceeding these values produces physically impossible poses and must be prevented in the IK/FK solver.

## 2.3 Primary Muscle Groups and EMG Channel Mapping

Reference: Kendall et al., Muscles: Testing and Function, 5th Ed. The forearm myoband captures EMG from 8 electrode sites spaced equidistantly around the forearm circumference, approximately 5 cm distal to the elbow.

The following mapping is a standard approximation used in prosthetics research (see Phinyomark et al., 2012, doi:10.1016/j.eswa.2011.10.013):

| Ch. | Primary Muscle | Secondary Muscle | Primary Movement Contribution |
| --- | --- | --- | --- |
| 0 | Flexor Digitorum Superficialis | Flex. Digit. Profundus | Finger flexion (PIP2–5) |
| 1 | Flexor Carpi Radialis | Flex. Carpi Ulnaris | Wrist flexion, radial deviation |
| 2 | Extensor Digitorum Communis | Ext. Digiti Minimi | Finger extension (MCP2–5) |
| 3 | Extensor Carpi Radialis | Ext. Carpi Ulnaris | Wrist extension, radial deviation |
| 4 | Flexor Pollicis Longus | Abductor Pollicis Longus | Thumb flexion, opposition |
| 5 | Extensor Pollicis Longus | Extensor Pollicis Brevis | Thumb extension, retropulsion |
| 6 | Pronator Teres | Pronator Quadratus | Forearm pronation |
| 7 | Supinator | Biceps Brachii (distal) | Forearm supination |

# 3. Myosignal File Format (.myo.csv)

The agent must define, implement, and document a custom file format for importing EMG recordings. The format is a UTF-8 encoded CSV file with the extension .myo.csv.

## 3.1 File Structure

The file consists of two sections: a metadata header block and a data block.

### 3.1.1 Metadata Header

All metadata lines begin with a # character and must appear before the data header row. Required fields:

| Key | Type | Description |
| --- | --- | --- |
| # FORMAT_VERSION | string | Always '1.0' for this specification |
| # SAMPLE_RATE_HZ | integer | Samples per second per channel. Standard Myo: 200 |
| # NUM_CHANNELS | integer | Must be 8 for this project |
| # SUBJECT_ID | string | Anonymised subject identifier (e.g. SYNTH_001) |
| # GESTURE_LABELS | string[] | Comma-separated list of gesture names in recording |
| # DURATION_S | float | Total recording duration in seconds |
| # NOTES | string | Optional. Free-text notes about the recording |

### 3.1.2 Data Block

Immediately following the header, a single column-name row defines the data schema. Data rows follow with one row per sample. Columns:

| Column Name | Type | Description |
| --- | --- | --- |
| timestamp_ms | float | Time offset in milliseconds from start of recording (0.0 at first sample) |
| ch0–ch7 | float | Normalised EMG amplitude per channel. Range: [-1.0, 1.0]. Raw mV values must be normalised to MVC (Maximum Voluntary Contraction) before import. |
| gesture_label | string | Gesture name active at this sample. Use 'rest' for no gesture. Values must match GESTURE_LABELS header. |
| gesture_id | integer | Integer ID of gesture. 0 = rest. Matches index in GESTURE_LABELS header (1-based). |

### 3.1.3 Sample File

The following is a minimal synthetic .myo.csv example (10 samples shown). The agent must include a Python script (generate_synthetic_myo.py) that produces a full 10-second synthetic recording:

# FORMAT_VERSION,1.0
# SAMPLE_RATE_HZ,200
# NUM_CHANNELS,8
# SUBJECT_ID,SYNTH_001
# GESTURE_LABELS,rest,fist,open_hand,pinch,point,thumbs_up,peace,ok
# DURATION_S,10.0
# NOTES,Synthetic data generated for unit testing
timestamp_ms,ch0,ch1,ch2,ch3,ch4,ch5,ch6,ch7,gesture_label,gesture_id
0.0,0.021,-0.013,0.008,0.005,0.003,-0.007,0.011,-0.004,rest,0
5.0,0.019,-0.015,0.010,0.004,0.002,-0.006,0.009,-0.005,rest,0
10.0,0.412,0.389,0.021,0.018,0.095,0.034,0.012,0.008,fist,1
15.0,0.578,0.501,0.025,0.022,0.112,0.041,0.014,0.010,fist,1
20.0,0.021,0.018,0.418,0.403,0.008,0.421,0.011,0.009,open_hand,2

## 3.2 Synthetic Data Generator Script

The agent must create Assets/Scripts/Tools/GenerateSyntheticMyo.py (run outside Unity, requires numpy) that outputs a valid .myo.csv file with the following characteristics:

- Duration: 10 seconds at 200 Hz (2000 samples total).

- 8 gesture classes + rest, each held for 1 second with 0.1s inter-gesture rest.

- Gesture EMG patterns: per-channel Gaussian activations centred on the channel-muscle mapping table (Section 2.3). Use numpy.random.normal with gesture-specific means and sigma=0.08.

- Rest segments: all channels ~ N(0, 0.02) white noise.

- Smooth transitions: apply a 20-sample Hanning window ramp between gesture and rest segments.

# 4. Unity Project Structure

Unity version: 6000.3.10f1. Reference: https://docs.unity3d.com/6000.0/Documentation/Manual/ProjectStructure.html

The agent must create the following directory layout under the Assets/ folder:

Assets/
├── Scripts/
│   ├── Core/
│   │   ├── MyoFileParser.cs
│   │   ├── EMGDataBuffer.cs
│   │   └── PlaybackController.cs
│   ├── Hand/
│   │   ├── HandRig.cs
│   │   ├── FingerController.cs
│   │   └── GesturePoser.cs
│   ├── Classification/
│   │   ├── IGestureClassifier.cs         (interface)
│   │   ├── MLGestureClassifier.cs        (Sentis/ONNX)
│   │   ├── RuleBasedClassifier.cs        (fallback)
│   │   └── ClassifierFactory.cs
│   ├── UI/
│   │   └── PlaybackUI.cs
│   └── Tools/
│       └── GenerateSyntheticMyo.py
├── Models/
│   └── Hand/                             (hand FBX + materials)
├── ML/
│   └── GestureClassifier.onnx            (trained model asset)
├── StreamingAssets/
│   └── SampleData/
│       └── synthetic_001.myo.csv
├── UI/
│   └── PlaybackPanel.uxml
└── Scenes/
    └── HandDemo.unity

## 4.1 Required Unity Packages

The agent must add the following packages via the Package Manager (Window > Package Manager). Reference: https://docs.unity3d.com/6000.0/Documentation/Manual/upm-ui.html

| Package | Version | Purpose |
| --- | --- | --- |
| com.unity.sentis | 2.1.x (latest) | ONNX ML model inference at runtime |
| com.unity.animation.rigging | 1.3.x (latest) | Procedural bone constraint system for hand rig |
| com.unity.nuget.newtonsoft-json | 3.2.x | Optional: JSON config parsing |

# 5. Hand Model & Rig

## 5.1 Model Sourcing Strategy

The agent must source a suitable hand model using the following priority order:

- Check the Unity Asset Store for a free, rigged hand model with humanoid-compatible bone naming. Search query: 'rigged hand free humanoid'. Preferred asset: 'Low Poly Hand' or equivalent with MIT/free licence. Asset Store URL: https://assetstore.unity.com/

- If no suitable free asset is found, procedurally generate a simplified hand mesh using Unity's ProBuilder package (com.unity.probuilder, https://docs.unity3d.com/Packages/com.unity.probuilder@6.0/manual/). Build 15 capsule segments (one per phalanx) connected by Transform parents matching the bone hierarchy in Section 2.1.

- If importing an FBX, ensure the model is imported with the Humanoid Avatar configuration disabled — use Generic rig instead, and manually map bones to the HandRig.cs bone references.

## 5.2 Bone Naming Convention

Regardless of model source, the agent must rename or alias all hand bones to the following convention. This ensures HandRig.cs can reference bones by name without hard-coded paths:

| GameObject Name | Anatomical Name | Parent |
| --- | --- | --- |
| Hand_Root | Wrist | Forearm / scene root |
| Thumb_MC | MC1 | Hand_Root |
| Thumb_PP | PP1 | Thumb_MC |
| Thumb_DP | DP1 | Thumb_PP |
| Index_MC | MC2 | Hand_Root |
| Index_PP | PP2 | Index_MC |
| Index_MP | MP2 | Index_PP |
| Index_DP | DP2 | Index_MP |
| (repeat for Middle, Ring, Pinky) | MC3–5, PP–DP | Same pattern |

## 5.3 HandRig.cs — Implementation Spec

Reference: Unity Transform API — https://docs.unity3d.com/6000.0/Documentation/ScriptReference/Transform.html

Reference: Animation Rigging — https://docs.unity3d.com/Packages/com.unity.animation.rigging@1.3/manual/

HandRig.cs must expose the following public fields (serialised in Inspector):

- Transform[] fingerBones — array of all 14 phalanx Transform references, ordered: Thumb_PP, Thumb_DP, Index_PP, Index_MP, Index_DP, Middle_PP, Middle_MP, Middle_DP, Ring_PP, Ring_MP, Ring_DP, Pinky_PP, Pinky_MP, Pinky_DP.

- float poseBlendSpeed — lerp speed (default: 8.0f). Controls how fast bones animate to target pose.

HandRig.cs must expose the following public method:

public void ApplyGesturePose(GesturePose pose, float blendWeight = 1.0f)

Where GesturePose is a ScriptableObject (Assets/Scripts/Hand/GesturePose.cs) containing a Quaternion[] targetRotations array of length 14, one per fingerBones entry.

Internally, ApplyGesturePose must use Quaternion.Slerp (https://docs.unity3d.com/6000.0/Documentation/ScriptReference/Quaternion.Slerp.html) driven by Time.deltaTime * poseBlendSpeed to smoothly interpolate each bone from current rotation to target. Clamping to anatomical ROM (Section 2.2) must be applied after Slerp, before assignment.

# 6. EMG Signal Pipeline

## 6.1 MyoFileParser.cs

Responsibilities: parse a .myo.csv file from disk into an in-memory EMGDataBuffer. Must validate header metadata fields and throw a descriptive exception if any required field is missing or malformed.

Reference: System.IO.StreamReader — https://learn.microsoft.com/en-us/dotnet/api/system.io.streamreader

Parsing rules:

- Skip lines starting with #. Extract key/value by splitting on the first comma after the key token.

- The column header row is the first non-# line. Validate it matches the expected schema exactly.

- Each data row: parse timestamp_ms as double, ch0–ch7 as float[], gesture_label as string, gesture_id as int.

- Clamp all channel values to [-1.0, 1.0] on import. Log a warning (Debug.LogWarning) if any value exceeds this range.

## 6.2 EMGDataBuffer.cs

A plain C# data container. Fields:

- double[] Timestamps — millisecond offsets.

- float[,] Channels — shape [sampleCount, 8].

- string[] GestureLabels — per-sample gesture name.

- int[] GestureIds — per-sample gesture integer.

- int SampleRate — from file metadata.

- int SampleCount — total rows parsed.

Must implement a method: EMGSample GetSampleAt(int index) returning a struct with all per-sample fields. Must implement GetWindowAt(int centerIndex, int windowSize) returning a float[windowSize, 8] slice (used by the classifier, Section 7).

## 6.3 Signal Preprocessing

Reference: EMG signal processing best practices — Merletti & Cerone, 2020, doi:10.3390/s20061566

Before classification, each window must undergo the following preprocessing steps applied in order. The agent must implement these as static methods in a class EMGPreprocessor.cs:

- Band-pass filter: Apply a 4th-order Butterworth filter, passband 20–450 Hz. Implement using a direct-form II transposed IIR filter. Pre-compute coefficients for the target sample rate at startup. Reference: https://en.wikipedia.org/wiki/Butterworth_filter

- Full-wave rectification: abs(sample) for all values.

- RMS envelope: sliding window RMS with window length = sample_rate / 10 (20 samples at 200 Hz).

- Normalisation: divide by per-channel maximum across the entire loaded buffer (computed once on file load).

NOTE: If the sample rate from the file header is not 200 Hz, the agent must recompute the Butterworth coefficients for the actual sample rate. Do not hard-code 200 Hz coefficients.

# 7. Gesture Classification System

## 7.1 Architecture Overview

The classification system uses a strategy pattern with a shared interface IGestureClassifier. The active strategy is selected by ClassifierFactory.cs at startup based on whether a valid ONNX model asset is present.

| Class | Strategy | When Active |
| --- | --- | --- |
| MLGestureClassifier | ML (primary) | Assets/ML/GestureClassifier.onnx exists and loads without error |
| RuleBasedClassifier | Rules (fallback) | ONNX asset missing, load fails, or inference throws exception |

## 7.2 IGestureClassifier Interface

public interface IGestureClassifier
{
    GestureResult Classify(float[,] window);  // window: [windowSize, 8]
    string[] GestureNames { get; }
    void Dispose();
}
public struct GestureResult
{
    public int GestureId;
    public string GestureName;
    public float Confidence;        // 0.0–1.0
    public float[] AllProbabilities; // length = num_gestures + 1 (rest)
}

## 7.3 MLGestureClassifier.cs — ONNX / Unity Sentis

Reference: Unity Sentis documentation — https://docs.unity3d.com/Packages/com.unity.sentis@2.1/manual/

Reference: Unity Sentis API — https://docs.unity3d.com/Packages/com.unity.sentis@2.1/api/

Implementation requirements:

- Load the model using ModelLoader.Load(modelAsset) where modelAsset is a Unity.Sentis.ModelAsset Inspector reference.

- Use BackendType.GPUCompute if SystemInfo.supportsComputeShaders, else BackendType.CPU. Reference: https://docs.unity3d.com/Packages/com.unity.sentis@2.1/api/Unity.Sentis.BackendType.html

- Input tensor shape: [1, windowSize, 8] (batch=1, timesteps, channels). Construct via TensorFloat.AllocNoData.

- Run inference via worker.Schedule(inputTensor). Retrieve output via worker.PeekOutput() as TensorFloat.

- Output tensor shape: [1, num_classes]. Apply softmax if not already in model. Return argmax as GestureId.

- Implement IDisposable. Dispose worker and tensors in Dispose(). The agent must never leak Sentis tensors — use try/finally blocks.

### 7.3.1 ONNX Model Specification

The agent must produce a training script (Assets/Scripts/Tools/train_gesture_model.py) using scikit-learn and skl2onnx. The model architecture:

- Input: flattened window features — shape (n_samples, windowSize * 8).

- Feature extraction: compute these 4 time-domain features per channel per window: Mean Absolute Value (MAV), Root Mean Square (RMS), Zero Crossing Rate (ZCR), Waveform Length (WL). Total feature vector length: 4 * 8 = 32 features.

- Classifier: MLPClassifier with hidden_layer_sizes=(128, 64), activation='relu', max_iter=500.

- Training data: NinaPro DB5 (https://ninapro.hevs.ch/instructions/DB5.html) or the synthetic generator from Section 3.2.

- Export via skl2onnx.convert_sklearn, opset version 17. Reference: https://onnx.ai/sklearn-onnx/

- Save to Assets/ML/GestureClassifier.onnx.

## 7.4 RuleBasedClassifier.cs — Fallback

When the ML model is unavailable, a rule-based classifier uses channel activation thresholds derived from the muscle-channel mapping in Section 2.3.

Rules operate on the RMS envelope of the current window (preprocessed per Section 6.3). The following threshold table must be implemented as a static readonly dictionary:

| Gesture | Primary Channel Condition | Secondary Condition |
| --- | --- | --- |
| rest (0) | ALL channels < 0.15 | — |
| fist (1) | ch0 > 0.4 AND ch1 > 0.35 | ch2 < 0.2 (not extending) |
| open_hand (2) | ch2 > 0.4 AND ch3 > 0.35 | ch0 < 0.2 (not flexing) |
| pinch (3) | ch4 > 0.4 AND ch0 > 0.3 | ch2 < 0.25 |
| point (4) | ch2 > 0.35 AND ch0 > 0.3 | ch4 < 0.2 (thumb relaxed) |
| thumbs_up (5) | ch4 > 0.45 AND ch5 > 0.3 | ch0 < 0.2 |
| peace (6) | ch2 > 0.4 AND ch0 > 0.25 | ch4 < 0.2 |
| ok (7) | ch4 > 0.4 AND ch0 > 0.3 | ch2 > 0.2 (index extended) |

Confidence for the rule-based classifier is computed as the ratio of activated channels that match the winning gesture rule, normalised to [0, 1]. AllProbabilities is set to uniform for non-winning classes.

# 8. Gesture Pose Library

The agent must create a GesturePoseLibrary ScriptableObject (Assets/Scripts/Hand/GesturePoseLibrary.cs) that stores one GesturePose per gesture class. Each GesturePose holds the 14 target Quaternion rotations for the finger bones in the pose.

The following Euler angles (localRotation relative to T-pose / anatomical rest) define the pose library. Axes convention: X = flexion (+) / extension (-), Y = abduction / adduction, Z = axial rotation. Reference: Section 2.2 ROM table.

| Gesture | Th_PP | Th_DP | Idx_PP | Idx_MP | Idx_DP | Mid_PP | Rng_PP | Pnk_PP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| rest | 0° | 0° | 0° | 0° | 0° | 0° | 0° | 0° |
| fist | 45°X | 60°X | 85°X | 90°X | 70°X | 85°X | 85°X | 85°X |
| open_hand | −10°X | 0° | −5°X | 0° | 0° | −5°X | −5°X | −5°X |
| pinch | 40°X | 55°X | 50°X | 55°X | 50°X | 85°X | 85°X | 85°X |
| point | 20°X | 30°X | 0° | 0° | 0° | 85°X | 85°X | 85°X |

NOTE: The middle, ring, and pinky phalanges MP and DP are omitted from this table for brevity. The agent must extrapolate them proportionally: MP = 0.9 × PP flexion, DP = 0.7 × PP flexion, based on anatomical coupling ratios from Hirt et al., Hand and Wrist Anatomy and Biomechanics (doi:10.1055/b-006-161455).

# 9. Playback Controller

## 9.1 PlaybackController.cs

Reference: Unity Coroutines — https://docs.unity3d.com/6000.0/Documentation/Manual/Coroutines.html

Reference: Unity Time — https://docs.unity3d.com/6000.0/Documentation/ScriptReference/Time.html

PlaybackController.cs is the central MonoBehaviour coordinating file parsing, classification, and hand animation. It must implement the following state machine:

| State | Description |
| --- | --- |
| Idle | No file loaded. UI shows import button only. |
| Loaded | File parsed, buffer populated, classifier initialised. Playback cursor at sample 0. |
| Playing | Advancing cursor at sample rate. Each frame: get window → classify → apply pose. |
| Paused | Cursor frozen. Hand holds last pose. |
| Error | Parse or classifier error. UI shows error message string. |

The playback cursor must advance in real time using accumulator-based timing (not coroutine WaitForSeconds). Each Update():

_accumulator += Time.deltaTime;
while (_accumulator >= _sampleInterval)
{
    AdvanceSample();
    _accumulator -= _sampleInterval;
}

Where _sampleInterval = 1.0f / buffer.SampleRate.

## 9.2 Step Navigation

A 'step' is defined as a gesture transition boundary — the sample index where gesture_id changes. PlaybackController must pre-compute a List<int> _stepBoundaries on file load by scanning the GestureIds array for transitions.

- NextStep(): advance cursor to _stepBoundaries[currentStepIndex + 1]. Clamp to last step.

- PreviousStep(): rewind to _stepBoundaries[currentStepIndex - 1]. Clamp to 0.

# 10. User Interface

## 10.1 UI Toolkit Implementation

Reference: Unity UI Toolkit — https://docs.unity3d.com/6000.0/Documentation/Manual/UIElements.html

Reference: UXML schema — https://docs.unity3d.com/6000.0/Documentation/Manual/UIE-UXML-element-reference.html

The UI is implemented as a single UI Toolkit panel (Assets/UI/PlaybackPanel.uxml) rendered via a UIDocument component on a Canvas GameObject in the HandDemo scene. The panel must be positioned in the bottom-left corner of the screen with a semi-transparent dark background (rgba 0,0,0,0.6).

## 10.2 Required Controls

| Control | Type | Behaviour |
| --- | --- | --- |
| Import File | Button | Opens RuntimeFileBrowser or Unity's built-in NativeFilePicker. Loads .myo.csv into PlaybackController. |
| Play / Pause | Toggle Button | Switches PlaybackController between Playing and Paused states. Label changes to reflect current state. |
| Previous Step | Button | Calls PlaybackController.PreviousStep(). Disabled in Idle/Error states. |
| Next Step | Button | Calls PlaybackController.NextStep(). Disabled in Idle/Error states. |
| Progress Bar | ProgressBar | Displays current sample / total samples as 0.0–1.0 fill. Updated every frame during Playing. |
| Gesture Label | Label | Displays current gesture name and confidence percentage (e.g. 'fist — 91%'). |
| Classifier Mode | Label | Displays 'ML Model' or 'Rule-Based (fallback)' to indicate active classifier. |
| Error Message | Label | Visible only in Error state. Displays exception message in red. |

## 10.3 File Browser

Unity 6 does not include a built-in cross-platform file picker for runtime builds. The agent must implement the following platform-conditional approach:

- Editor only: Use UnityEditor.EditorUtility.OpenFilePanel (https://docs.unity3d.com/6000.0/Documentation/ScriptReference/EditorUtility.OpenFilePanel.html). Wrap in #if UNITY_EDITOR directive.

- Standalone builds: Implement a simple in-scene file browser using UI Toolkit that lists files from Application.persistentDataPath (https://docs.unity3d.com/6000.0/Documentation/ScriptReference/Application-persistentDataPath.html) and a user-configurable watched directory (set via PlayerPrefs).

# 11. Scene Setup — HandDemo.unity

The agent must configure the HandDemo scene with the following GameObject hierarchy:

HandDemo (Scene)
├── Lighting/
│   ├── Directional Light  (intensity 1.2, rotation 50/-30/0)
│   └── Ambient Light      (Color: soft grey, intensity 0.4)
├── Camera/
│   └── Main Camera        (position: 0, 0.1, -0.35 | look at Hand_Root)
├── Hand/
│   ├── Hand_Root
│   │   ├── [All finger bone hierarchy per Section 5.2]
│   ├── HandRig             [HandRig.cs component]
│   └── GesturePoser        [GesturePoser.cs component]
├── Controllers/
│   └── PlaybackController  [PlaybackController.cs component]
└── UI/
    └── UIDocument          [UIDocument component, sourceAsset: PlaybackPanel.uxml]

Camera setup: the Main Camera must frame the hand comfortably. Use an orthographic projection (size 0.15) or perspective (FOV 40°, position 35 cm from Hand_Root). The hand should occupy ~60% of the screen vertically.

# 12. Testing Requirements

## 12.1 Unit Tests

Reference: Unity Test Framework — https://docs.unity3d.com/Packages/com.unity.test-framework@1.4/manual/

The agent must create an EditMode test assembly (Tests/EditMode/) with test classes for:

- MyoFileParserTests — valid file parse, missing header field, out-of-range channel value, empty file.

- EMGPreprocessorTests — band-pass filter output shape, RMS envelope correctness (compare against numpy reference values in comments), normalisation bounds.

- RuleBasedClassifierTests — each gesture rule fires correctly given synthetic input matching threshold conditions; rest fires when all channels below threshold.

- GesturePoseLibraryTests — all 8 gesture poses exist; no Quaternion is identity except rest; all rotations within anatomical ROM (Section 2.2).

## 12.2 Integration Test

A PlayMode test (Tests/PlayMode/PlaybackIntegrationTest.cs) must:

- Load the synthetic_001.myo.csv from StreamingAssets.

- Start playback.

- Wait 3 seconds (yield return new WaitForSeconds(3f)).

- Assert that at least 3 distinct gesture IDs have been classified.

- Assert that HandRig.cs finger bone rotations are non-identity (hand has moved).

# 13. Deliverables Checklist

The agent must produce all of the following. This list serves as the acceptance criteria:

| # | Deliverable |
| --- | --- |
| 1 | Unity project openable in Unity 6000.3.10f1 with no compile errors. |
| 2 | HandDemo.unity scene with all GameObjects per Section 11. |
| 3 | Hand model (sourced or procedural) with correctly named bone hierarchy per Section 5.2. |
| 4 | All C# scripts per Section 4 directory layout, compiling without warnings. |
| 5 | GesturePoseLibrary ScriptableObject asset with all 8 gesture poses configured per Section 8. |
| 6 | PlaybackPanel.uxml with all controls per Section 10.2. |
| 7 | generate_synthetic_myo.py producing a valid .myo.csv file when run. |
| 8 | synthetic_001.myo.csv present in StreamingAssets/SampleData/. |
| 9 | train_gesture_model.py with ONNX export per Section 7.3.1. |
| 10 | GestureClassifier.onnx present in Assets/ML/ (trained on synthetic data if NinaPro unavailable). |
| 11 | All unit and integration tests passing per Section 12. |
| 12 | README.md in project root: setup instructions, package install steps, how to run tests, how to generate synthetic data. |

# 14. Agent Implementation Constraints

- The agent must not use any deprecated Unity APIs. All Unity API calls must be verified against the Unity 6000.0 ScriptReference at https://docs.unity3d.com/6000.0/Documentation/ScriptReference/ before use.

- The agent must not use Unity's legacy OnGUI system for UI. All UI is implemented via UI Toolkit as specified in Section 10.

- The agent must not use UnityEngine.Animator for hand poses. All bone rotation is applied directly via Transform.localRotation as specified in Section 5.3.

- All C# code must target .NET Standard 2.1 (Unity 6 default). No unsafe code blocks.

- All ML inference must use Unity Sentis. No external ML runtime DLLs.

- The agent must never hard-code absolute file paths. Use Application.streamingAssetsPath, Application.persistentDataPath, or Path.Combine for all file operations.

- The agent must handle all exceptions from file I/O and ML inference gracefully and transition to the Error state (Section 9.1) with a descriptive message.

- Sentis tensor allocations must be disposed after every inference call. Memory leaks from undisposed tensors will cause the integration test to fail.

# 15. Glossary

| Term | Definition |
| --- | --- |
| CMC | Carpometacarpal joint — joint between carpal bones and metacarpals. |
| DIP | Distal interphalangeal joint — between middle and distal phalanges. |
| DP | Distal Phalanx — the tip bone of a finger. |
| EMG | Electromyography — electrical measurement of muscle activation. |
| IP | Interphalangeal joint (thumb only). |
| MAV | Mean Absolute Value — time-domain EMG feature. |
| MC | Metacarpal bone. |
| MCP | Metacarpophalangeal joint — knuckle joint. |
| MP | Middle Phalanx. |
| MVC | Maximum Voluntary Contraction — used for EMG normalisation. |
| ONNX | Open Neural Network Exchange — cross-platform ML model format. |
| PIP | Proximal interphalangeal joint — between proximal and middle phalanges. |
| PP | Proximal Phalanx. |
| RMS | Root Mean Square — statistical measure of signal magnitude. |
| ROM | Range of Motion — anatomical joint movement limits. |
| Sentis | Unity's runtime ML inference engine for ONNX models. |
| WL | Waveform Length — sum of absolute differences between consecutive EMG samples. |
| ZCR | Zero Crossing Rate — number of times EMG signal crosses zero per window. |